Create original component library with LCo

March 2002

(c) Copyright OKADA Hitoshi(Suigyodo), 2002

1. Start LCo

2. Drawing pattern

When you create a component which has bit pattern body like below, you need to draw bit pattern before definition of pins.

When create a component without bit pattern which  consists of a simple sqare body and some pins, see chapter 3.

2-1. Dispaly Edit pattern window

On the Pattern menu, click the Edit pattern.

2-2. Create a new pattern

On the Edit menu, click the New pattern and  New pattern dialog will appear.

In the dialog, type the name of the pattern.

2-3.Set pattern size

On the Edit menu, click the Set pattern size and  Pattern size dialog will appear.

Enter the size of the pattern.

As the unit, it uses the grid of BSch sheet .

Actual size of  3 is 31(=3 x 10 +1)pixels.

2-4. Draw pattern

Use the left button to put pixels or use right button to delete pixels when you draw with the Freehand-tool, Line-tool, or Circle-tool.

2-5.Close Edit pattern window

3. Definition of pins

3-1. Create a new component

On the Edit menu in LCo main window,click the New component.

In this dialog, type the name of the component.

3-2. Set the attribute of the component

On the Edit menu, click the Attribute.

Setup each attribute and click OK.

Block:  If you want to create a component which has some equivalent blocks in a single package like 7400 standard logic, set the number of  blocks in this edit box.

3-3. Definition of pins

(1)Click the end of  a pin. It moves focus mark to the pin.

(2)Setup the pin-type, the value, the number and so on.

(3)Click the Update pin button.

4. Save file

On the File menu, click Save as. And type the library name.

File type:  You can choose either ".LB3" or ".LIB" as file type of  the library. But it isn't recommended to save as ".LIB". ".LIB" is obsolate.